TDMA 5.8G 900Mbps Wireless Bridge

Model: WB5acL8-Ex

Description:

WB5acL8-Ex is a high-performance, high-speed outdoor long-distance wireless bridge. It has the characteristics of long transmission distance, strong penetration ability, and strong anti-interference ability..

Main Features:

- The use of specialized TVS lightning protection devices enhances the product's ability to withstand harsh environments.

- Built in 18dBi dual polarization directional antenna, up to 8km for PTP/ PTmP distance.

- Support non-standard frequency points: 4.9-6.1GHz.

- Support 5, 10MHz narrowband operating bandwidth

- Support ATPC, IPv6, TDMA.

Hardware Specification:

| Model | WB5acL8-Ex |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CPU | QCA9563+QCA9882+QCA8334 |  |  |  |  |
| Flash | SPI NOR 16MB |  |  |  |  |
| DDR | DDR2-128MB |  |  |  |  |
| 5G frequency | 5.150GHz～5.850GHz |  |  |  |  |
| 5G WIFI protocol | 802.11 a / n /ac |  |  |  |  |
| Max rate | 900Mbps |  |  |  |  |
| Antenna | Internal 18dBi directional panel antenna |  |  |  |  |
| 5G TX Power | 802.11a | 54M | 23±2dBm | 6M | 26±2dBm |
|  | 802.11n HT20 | MCS7 | 22±2dBm | MCS0 | 26±2dBm |
|  | 802.11n HT40 | MCS7 | 22±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT20 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
| 5G Receiving Sensitivity | 802.11ac VHT40 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT80 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
|  | 802.11a | 54M | -75dBm | 6M | -92dBm |
|  | 802.11n HT20 | MCS7 | -72dBm | MCS0 | -90dBm |
| 5G EVM | 802.11a_54M: ≤-25 dB；802.11n_MCS7: ≤-28 dB；802.11AC_MCS9:≤-32dB |  |  |  |  |
| ppm | ±20ppm |  |  |  |  |
| WAN | 1*10/100/1000M WAN, support 24V~48V passive POE |  |  |  |  |
| LAN | 1*10/100/1000M LAN |  |  |  |  |
| Power | PoE 24V passive POE, DC 12~24V |  |  |  |  |
| Max Power Consumption | <10W |  |  |  |  |
| Reset | Press 2-5 seconds for pairing, 10seconds for factory reseting. |  |  |  |  |
| Indicators | SYS, WAN, LAN, Signal*5 |  |  |  |  |
| Dimension | 268mm*115mm*45mm |  |  |  |  |
| Weight | 0.48kg |  |  |  |  |
| Temperature | Working: -30°C to 55°C；Storage: -40°C to 70°C；Humidity: 5％～95％(Non-Condensing) |  |  |  |  |
| IP Grade | IP65 |  |  |  |  |
| ESD | Air:±8K, Touch:±6K |  |  |  |  |
| Electrical Surge | Common Mode: 4K, Differential Mode: 2K |  |  |  |  |

Firmware Specification

| Working mode | Access Point (Automatic WDS), Access Point (TDMA3), Site (WDS/TDMA3), Site (ARPNAT) |
| --- | --- |
| Wireless | Intelligent dynamic polling, automatic channel selection, automatic modulation mode selection, automatic transmission power control (ATPC) |
| Wireless Safety | WPA/WPA2 personal encryption, WPA/WPA2 enterprise encryption, WACL, user isolation |
| Wireless QoS | TDMA implements 4 queue priority |
| Network mode | IPv4 IPv6 |
| VLAN | Support VLAN and VLAN transparent transmission |
| WAN protocol | Static IP，DHCP client |
| Service | DHCP service, SNMP service, NTP client, route tracking, ping watchdog |
| Management | HTTP(S) GUI, SSH, SNMP read, WNMS, Telnet |
| Built Tool | Site survey, connection testing, antenna calibration |

Antenna Specification

| Horizontal polarization |  | Vertical polarization |  |
| --- | --- | --- | --- |

| Built in Antenna |  |
| --- | --- |
| Working frequency | 5180 ~5850 MHz |
| Gain | 18 dBi |
| Polarization mode | Vertical/horizontal dual polarization |
| Horizontal beam angle | 30° |
| Vertical beam angle | 15° |

Interface:

Dimension:

Accessory

| CPE | 1 |
| --- | --- |
| PoE adapter | 1 |
| Gift Box | 1 |