Embedded 4G Router Module

User's manual

目录

I. Interface Description	3

Interface description	3

1. Connect the device	4

2. Log in to the router	5

III. Working Mode	6

2. Standard routing model	8

4. Wireless AP + Client Mode	13

IV. LAN Settings	14

V. Wireless WIFI Settings	15

1. Wireless basic settings	15

2. Wireless security settings	16

3. List of wireless users	16

4. Wireless MAC filtering	17

VI. Network Security	17

1. The firewall is open	17

2. Web address site control	18

3. IP access control	18

VII. System Services	20

2. DMZ settings	21

3. Serial port service	22

VIII. Equipment Management	22

1. Equipment management	22

2. Software upgrade	24

3. Restore the factory values	25

4. Password management	25

5. Backup, restore	26

# I. Interface Description

# Interface description

| Project | Parameter |
| --- | --- |
| WAN port (ETH0) | One 10/100M adaptive Wan port with built-in isolation, supporting auto rollover (Auto MDI/MDIX), LAN port in 4G mode |
| LAN port (ETH4) | One 10/100M adaptive LAN port, supporting automatic rollover (Auto MDI/MDIX) |
| String Mouth | 2 serial ports, UART0 is console, UART1 is data transmission port |
| USB port (PCIE) | USB 2.0 port, connected to 4G module. |
| Indicator light | 3 LEDs (SYS, 4G, WLAN) |
| Antenna interface | 2 Generation I I-PEX Block |
| Power connector | 3.3-12V /2A ±0.2V |

Indicator light status description

| Name | Status | Description |
| --- | --- | --- |
| WLAN | Flicker | Wireless WIFI is on |
|  | Put out | Wireless WIFI is not turned on |
| 4G | Move | Flash after power-on, indicating that the dialing process is in progress |
|  | Chang Liang | Always on after power-on, indicating successful dialing |
|  | Put out | If it turns off after power-on, it means that the LTE module or SIM card or non-4G working mode is not recognized. |
| System light (SYS) | Put out | Power on and off indicates that the power supply is abnormal or the system is not started. |
|  | Slow flash | After power-on, the system flashes slowly, indicating that the system is running normally. |

II. Setting preparation

- Connect the device

You can connect your device to the router by following these steps.

1) Wired connection

Wired to the LAN port of the router, set the IP address of the computer

Before accessing the router Web management settings page, we recommend that you set your computer to Obtain an IP Address Automatically and Obtain a DNS Server Address Automatically so that the router assigns IP addresses automatically. If you need to assign a static IP address to the computer, you need to set the IP address of the computer and the IP address of the LAN port of the router in the same subnet (the default IP address of LAN port of the router is 192.168.10.1, and the subnet mask is 255.255.255.0).

2) Connect via WiFi

The computer searches for the wireless network connection of the wireless router (the default wireless name is the router model + the last four digits of MAC), and then clicks the'Connect 'button to establish the connection. The wireless password is: 12345678.

2.Verify connectivity between the computer and the router

When your computer's network icon shows that you are connected, you have successfully obtained an IP address. To ensure successful connectivity, use the Ping command to confirm.

Method: Open the cmd command line window and execute

the ping command: Ping 192.168.10.1 If the screen 	displays as follows, it indicates that the computer 	has 	successfully established a connection with the router.

- Log in to the router

Next, you will log in to the router Web administration settings page.

Enter in the Web browser address barhttp://192.168.10.1Enter the login user name and password in the pop-up box.

When logging in for the first time, please enter the default user name: admin and password: admin.

After successful login, enter the Web management settings page. At this time, you can set and manage the router. In the current status options page, you can view the Wan port, LAN port, 3G/4G status and other information.

III. Working Mode

The M400D offers 4 operating modes: 3G/4G wireless routing mode (default), standard wireless routing mode, wireless AP + wireless client bridging mode, and wireless AP + wireless client mode:

1.3G/4G Wireless Routing Mode

The M400D defaults to 3G/4G wireless routing mode (i.e. 3G/4G wireless Internet access to wired or WIFI output). After the 3G/4G tariff card is inserted and powered on, the router will automatically identify the 3G/4G network dial-up Internet access.

You can also define the network operator you need to select in the 3G/4G setting option page, such as China Mobile, China Unicom, China Telecom, and APN card.

| Operator | 3G/4G network | APN | Dialing number | User name | Password |
| --- | --- | --- | --- | --- | --- |
| China Mobile | TD-SCDMA TDD-LTE | cmnet | * 99 * * * 1 # or * 99 # Or * 98 * 1 # | card | card |
| China Telecom | CDMA2000 FDD-LTE | ctnet | #777 | card | card |
| China Unicom | WCDMA FDD-LTE | 3gnet | *99# | Empty | Empty |

- Standard routing model

In the standard routing mode, the ETH0 port is a Wan port, which provides three Internet access modes: static IP address, dynamic IP address, and PPPoE dialing.

  - Static IP address access mode

After switching the working mode to the standard routing mode, select the static address in the WAN setting option page, input the IP address, network mask, gateway, DNS and other relevant parameters provided by the ISP (or the upper level router), and click OK.

View the connection status in the Current Status Options page.

  - Dynamic IP address network access mode

After switching the working mode to the standard routing mode, select the dynamic address in the WAN setting option page, and the router will automatically obtain the IP address of the WAN port. Click OK.

View the connection status in the Current Status Options page.

  - PPPoE dial-up mode

Connect the WAN port to ADSLModel, select the standard routing mode in the working mode menu option page of the WEB management interface of the router, then select PPPoE in the Internet access mode in the WAN setting menu option page, fill in the PPPoE user name and password provided by the ISP, and click OK.

View the connection status in the Current Status Options page.

- Wireless AP + Wireless Client Bridge Mode

In this mode, the M400D can be used as a bridging AP to bridge the WIFI signal of the previous wireless router. In this mode, the DHCP service function of the device is turned off. After successful bridging, the previous wireless router allocates IP to the LAN port or the device accessed by WIFI.

Set the local IP of the computer to 192.168.10.100, connect the LAN port through the network cable, enter the Web management interface of the router, and select the wireless AP + wireless client bridging mode in the working mode option page.

After the working mode is switched successfully, select LAN settings in the menu bar, and click to search for the wireless network to be bridged.

In the search window, you can see the valid wireless AP name in the current range, check the corresponding wireless AP, and record the security mode.

Select the corresponding security mode in Wireless Security, enter the selected wireless AP password, and click OK to bridge the wireless AP network of the previous route.

View the connection status in the Current Status Options page.

- Wireless AP + Client Mode

In this mode, the M400D is used as a terminal AP to connect the WIFI signal of the previous wireless router. In this mode, the DHCP service function of the device is enabled, and the IP is allocated to the LAN port or the device with WIFI access.

On the Operating Mode Options page, select Wireless AP + Client Mode.

After the working mode is switched successfully, select the wireless connection tab in the menu bar. In this mode, three Internet access modes are also provided:

static IP address, dynamic IP address, and PPPoE dialing. Select the corresponding Internet access mode, and click Search Wireless Network.

In the search window, you can see the valid wireless AP name in the current range, check the corresponding wireless AP, and record the security mode.

Select the corresponding security mode in Wireless Security, enter the selected wireless AP password, and click OK to bridge the wireless AP network of the previous route.

View the connection status in the Current Status Options page.

IV. LAN Settings

In the case of multiple router connections, if the LAN port IP address network segment is the same, it will cause IP address conflict and can not connect to the network, so it is necessary to modify the LAN port IP address to solve this problem.

Select LAN settings in the menu bar of the WEB management interface of the router, and modify the third group of IP address values in the LAN settings, ranging from 1 to 254, such as 192.168.1.1 and 192.168.254.1

DHCP service is used to dynamically allocate IP addresses for accessed devices. The range of the last group of values can be modified to limit the range of allocated IP addresses. If the connected device needs to be designated as static IP, this function will be turned off.

V. Wireless WIFI Settings

- Wireless basic settings

Select the 2.4G wireless option in the menu bar of the Web management interface of the router to configure the 2.4G wireless WIFI. On this page, you can turn on and off wireless, modify the wireless SSID name, and hide the SSID name.

- Wireless security settings

There are several types of wireless security modes, and you can choose different security modes according to your needs.

  - Deactivate

  - Open System (Unsecure)

  - WPA-PSK

  - WPA2-PSK (default, recommended)

  - WPA-PSK/WPA2-PSK (i.e. Mixed WPA-PSK and WPA2-PSK)

The default is WPA2-PSK high security mode, the encryption type is AES, and the cipher key format type is a single or combination of 8-63 digits of numbers, letters, and symbols.

- List of wireless users

You can view the devices that are currently connected to your wireless connection.

- Wireless MAC filtering

The MAC address of the terminal device can be added to the list, and the filtering function can be enabled to restrict the devices in the list to prohibit or allow access to the router through wireless WIFI.

VI. Network Security

- The firewall is open

The firewall is turned on by default. If the firewall needs to be turned off, uncheck it. To prevent PING from the WAN port, check the Prevent WAN Port PING function.

- Web address site control

You can add the website address or server IP address to the list, turn on the site control function, and the connected device will not be able to access the websites and servers in the list.

- IP access control

You can add IP address range and port range to the list, enable blocking or passing, and prohibit or allow the IP and port information in the restriction list to pass through the router.

VII. System Services

The M400D provides virtual port mapping, 232 serial port applications, and DMZ services. 1. Virtual Port Mapping Services

The M400D can be used as a network server to provide many service functions. It can be used as a virtual server only by filling in the external port, internal port and internal server IP address correctly. When externally accessing the WAN IP + external port, the router switches the access link map to the internal server IP.

- DMZ settings

It can be set to discard or redirect the port information to the specified DMZ host IP address when the externally accessed port information enters the router and no matching target is found.

- Serial port service

The external device is connected to the RS232 serial port of the router. After the router opens and configures the serial port service, the data information transmitted by the external device can be sent to the designated server.

  - Configure the server address and port number to receive data in client mode.

  - Set the communication parameters matching the external device in the COM configuration.

VIII. Equipment Management

- Equipment management

  - UPNP is enabled

By enabling the UPnP function, the router can realize NAT traversal: when computers in the local area network communicate with the Internet through the wireless router, the wireless router can automatically add and delete the NAT mapping table as required, thereby solving the problem that some traditional services (such as MSN voice and video) cannot traverse the NAT.

  - Remote management

You can enable remote login to manage the device through the Wan port, and enable telnet remote management to log in to the background system of the device.

  - Timing, timed restart

You can set a custom time to automatically restart the router, which is not enabled by default at 0.

  - Enable system logging

Enabling the system log function (as shown in the figure above) will record the running information of the system in detail, which is helpful to eliminate software anomalies.

Logs can be viewed and downloaded in System Information on the Current Status Options page.

- Software upgrade

Software upgrade allows you to update the latest version of the firmware to the router for more features and more stable performance.

Software upgrade steps are as follows:

- Click the < Browse > button to select the firmware (.bin file) to be upgraded.

- If you need to perform a factory reset after the upgrade, click the < Restore Factory Settings > button.

- Click the < Upgrade > button to start the upgrade.

Note: There are two conditions for upgrading and restoring the factory settings:

  - The version number changes; 2. Click the < Restore factory settings > button when upgrading.

- Restore the factory values

The wireless router will restart during the reset process.

Restoring to the factory settings will clear all the settings information of the wireless router and restore to the original state. This function is generally used when the device changes from one network environment to another different network environment, restores the device to the factory settings, and then resets it to be more suitable for the current networking.

Click < Restore factory settings > and confirm to restore factory settings.

- Password management

The default user name/password of the wireless router is admin. The user name cannot be modified, but the password can be modified. For the sake of security, it is recommended to modify the login password and keep the password information.

The setting steps are as follows:

- Enter the original password in the < Original password > text box; Enter the new password in the New Password text box,

Confirm the new password by re-entering it in the Confirm Password text box.

- Click the < OK > button to complete the password change.

- Backup, restore

- Back up the configuration information, click the < Backup > button on the setting information page, and download and save the configuration file to the local.

- To restore the configuration information, click the < Browse > button on the setting information page, select the saved backup file, and click < Restore >.